File 04_azone_fuel_power_cost.csv

April 9, 2026

Definition of the Input File

VisionEval (https://visioneval.github.io/docs/model-inputs.html#verspm-azone_fuel_power_cost.csv) defines azone_fuel_power_cost.csv as providing the retail cost of fuel and electricity used in the CalculateVehicleOperatingCost module. These costs are key inputs for estimating household vehicle operating expenses by azone.

Possible Scenario

One possible scenario in VisionEval based on this input file is to examine how changes in fuel and electricity prices influence household travel behavior, vehicle operating costs, and energy consumption. Adjustments to these prices may affect vehicle miles traveled, vehicle type choice (e.g., ICE vs. electric), and overall transportation expenditures. For example, increases in fuel prices may lead households to reduce discretionary travel, shift to more fuel-efficient vehicles, or increase adoption of alternative modes such as transit, walking, or biking. Lower fuel prices may encourage additional discretionary driving and higher vehicle miles traveled. Changes in electricity prices may similarly influence the relative cost of operating electric vehicles, potentially affecting the rate of EV adoption or usage.

Method Used for Virginia

Overview

The file compiles two primary cost components: retail fuel prices (gasoline and diesel) and electricity prices. These values are used to estimate per-mile operating costs for different vehicle types across all azones. The dataset includes both a base year and a forecast year for these two prices.

Initial Calculations

Fuel price data were obtained from the U.S. Energy Information Administration (EIA), specifically the Weekly Retail Gasoline and Diesel Prices dataset. Units for fuel prices are $/gallon, including taxes.

Electricity price data were sourced from the EIA’s Electric Sales, Revenue, and Average Price dataset. Units for electricity prices are $/kWh.

Fuel Costs

Fuel cost data reflect values for 2024. Because the file structure includes year-specific field names, both base year and future year values must be expressed in constant (2024) dollars.

To estimate the target year values, historical fuel price trends and price index data from the model’s deflator (def) folder were used. While fuel prices fluctuate annually, the long-term trend is relatively stable (see below). Based on this observation, target year values were set equal to base year values in constant base year dollars. In short, we expect fuel cost growth to match overall inflation in the long run.

Fuel cost vs. Price Index (from def folder) (1999-2024).

Electricity Costs

Electricity price data reflect 2023 values. To align with the fuel cost base year, a 2024 value was projected using a 5-year compound annual growth rate (CAGR).

Base Year Projection Using 5-Year GAGR.

A similar approach to fuel costs was used for projecting 2045 values. Electricity price trends and deflator data were reviewed, and, given the consistent long-term trend (see below), target year values were set equal to base year values in constant base year dollars.
Electricity Cost Growth vs. Price Index Growth (2019-2024).

Sources

- VisionEval definition of the input file: https://visioneval.github.io/docs/model-inputs.html#verspm-azone_fuel_power_cost.csv

- VisionEval CalculateVehicleOperatingCost: https://github.com/visioneval/visioneval/blob/main/sources/modules/VETravelPerformance/inst/module_docs/CalculateVehicleOperatingCost.md

- U.S. Energy Information Administration – Weekly Retail Gasoline and Diesel Prices: https://www.eia.gov/dnav/pet/pet_pri_gnd_a_epmr_pte_dpgal_w.htm

- U.S. Energy Information Administration – Electric Sales, Revenue, and Average Price: https://www.eia.gov/electricity/sales_revenue_price/