Optional File 06_azone_hh_ave_veh_per_driver.csv

June 1, 2026

Definition of the Input File

This file does not appear in the VisionEval inputs (https://visioneval.github.io/docs/model-inputs.html) and as of now does not appear to affect VisionEval results.  Execution of the multimomodal module and then summing the attributes UrbanHhDvmt, TownHhDvmt, and RuralHhDvmt (found in a test file Marea_SayedMM_2045) did not show any differences for the cases of 1.1 vehicles per household, 0.1 vehicles per household, and running the file entirely.  The only mention of azone_hh_ave_veh_per_driver.csv in official VisionEval documentation is the following: 

“Users can also use the azone_hh_ave_veh_per_driver.csv to reduce or increase vehicle ownership at the Azone level. It should be noted that this file inherently reduces the sensitivity of the AdjustVehicleOwnership module”

In the future, if this file were needed, it could be defined as shown below:  the average number of vehicles per driver by azone. This input is used to estimate vehicle availability within households and plays a role in determining travel behavior and vehicle ownership patterns.

Possible Scenario

One possible scenario for this input file is to examine how changes in vehicle availability per driver influence travel behavior, vehicle ownership patterns, and transportation demand. Higher vehicles per driver may indicate greater accessibility to personal vehicles, potentially increasing vehicle miles traveled and reducing reliance on alternative modes of transit. Lower vehicles per driver may reflect constraints on vehicle access, encouraging mode shifts or trip consolidation. Changes in this variable may also reflect broader societal trends such as urbanization, affordability constraints, or policy interventions aimed at reducing vehicle dependence.

Method Used for Virginia

Overview

The file was developed by combining data on the number of drivers and the number of vehicles for each azone. The ratio of vehicles to drivers was calculated to represent the average number of vehicles available per driver. Values were estimated for a base year and a future forecast year to support scenario modeling.

Initial Calculations

Driver counts were obtained from the Virginia Traffic Crash Facts report for 2023. Vehicle counts were obtained from the U.S. Census Bureau Table B25046: Aggregate Number of Vehicles Available by Tenure from the 2023 ACS 5-Year Estimates. The total number of vehicles was divided by the total number of drivers for each azone to calculate an initial 2023 ratio.

Forecast Assumptions

National trends suggest that the average number of vehicles per driver is gradually declining due to factors such as increased urbanization, improved access to alternative transportation modes, and rising vehicle costs. Based on this trend and limited availability of localized forecasts, base year values were estimated by multiplying the 2023 ratios by 0.985. 2045 values were estimated by multiplying the 2023 ratios by 0.85 to reflect a continued long-term decline.

Sources

- VisionEval mention: https://visioneval.github.io/docs/validation-and-troubleshooting.html

- Virginia Traffic Crash Facts Report (2023): https://www.dmv.virginia.gov/sites/default/files/documents/VA-traffic-crash-2024.pdf

- U.S. Census Bureau ACS Table B25046 (2023 5-Year Estimates): https://data.census.gov/table/ACSDT5Y2023.B25046