07_azone_hh_lttrk_prop

May 25, 2026

## Definition of the Input File

The file azone_lttrk_prop.csv specifies the proportion of household vehicles that are light trucks (pickup trucks, SUVs, and vans) within each Azone and year. This input is formally defined in the VisionEval documentation: https://visioneval.github.io/docs/model-inputs.html#azone_lttrk_prop where the main attribute is the proportion of household vehicles classified as light trucks within each analysis zone (Azone) for each modeled year. In the VisionEval framework, passenger vehicles are bifurcated into two primary categories based on EPA MOVES classifications: Autos (SourceType 21, including sedans and coupes) and Light Trucks (SourceType 31, including pickups, SUVs, and vans).

For example, the table below suggests that in year 2024, less than half (40.7%) of vehicles in the City of Charlottesville are light duty trucks or SUVs, with this percentage being unchanged in 2045.

### Possible Scenario

The proportion of light trucks is a critical driver of a region's environmental and economic profile because these vehicles typically possess different energy consumption rates and emission profiles compared to passenger cars. This file allows planners to test diverse scenarios, such as urban vehicle downsizing policies where dense areas like Alexandria City might see shifts in vehicle preference or rural demand shifts driven by functional needs in areas like Albemarle County.

For instance, a scenario might model a significant shift in consumer preference toward larger vehicles (SUVs and trucks) as they become increasingly electrified. In this case, the LtTrkProp would increase over time. By comparing a 2024 baseline to a 2045 projection, VisionEval can calculate the resulting impacts on greenhouse gas emissions, household transportation costs, and vehicle ownership decisions.

The document Task 7.2 Simplified Scenario Development guide available at A Playbook for Simplified Approaches for Creating Scenarios in VisionEval does not explicitly suggest that this input file lends itself to a what-if scenario.  However, the document notes that vehicles in general have grown heavier, and thus a policy that might affect the proportion of such vehicles, such as a tax based on vehicle weight, could thus be developed based on this input file.

### Method Used for Virginia Statewide

Base Year 2024

From the 2021 Report completed on VisionEval (22-R21.pdf), it was determined that, “It is possible to roughly estimate the proportion of household vehicles that are trucks by using vehicle populations periodically developed by the Virginia Department of Environmental Quality (Lewis-Cheatham, 2020) as part of the National Emissions Inventory.  These populations are used with the EPA MOVES model and the proportion can be determined from two of the 13 MOVES vehicle types:  passenger car (type 21) and passenger truck (31), the latter of which includes “many SUVs and minivans” (U.S. EPA, 2019).   This approach is thus only an estimate; also there is not a perfect alignment between EPA vehicle classifications and FHWA vehicle classifications (Ponticello, 2020) with the U.S. EPA (2019) defining “light duty vehicles” based on weight and emissions.  Further, it is also possible that some of these vehicles are for commercial uses rather than household uses.  That said, one method for determining the inputs for this file (azone_lttrk_prop.csv) is to use the vehicle populations from the NEI.  For example, in 2017, there were 467,631 passenger cars in Fairfax County (vehicle type 21 in the National Emissions Inventory) and 364,020 passenger trucks in Fairfax County (vehicle type 31), such that the proportion of trucks is 364,020 / (467,631  + 364,020) = 43.8% in 2017.  This proportion was presumed to remain constant two years later in 2019 and again in 2024.

Forecast Year 2045

For 2045, there are not Virginia specific forecasts but there are national or worldwide forecasts that suggest this ratio may not necessarily remain constant.  Voelk (2020) quotes an interviewee from IHS Markit who stated that in 2025, the “light-truck segment” (inclusive of SUVs and pickup trucks) was expected to increase from 72% to 78%.  A graphic published in Schuster (2018) forecast that the share of SUVs sold in the U.S. would increase from 50% in 2020 to 53% in 2025.  The International Energy Agency (2019) noted that worldwide use of oil attributed to growing popularity of SUVs could reduce the benefit of electrification between the present and 2040, although this worldwide forecast includes other countries where SUV use is lower than in the U.S. at present and thus has more room to increase (Schuster, 2018).

However, the passage of time (2021-2026) suggests that it is difficult to adequately apply these national forecasts to Virginia datasets.  Thus this file presumes 2045 and 2024 values are the same.  The user may, of course, change these values if better forecast information becomes available.

Sources

International Energy Agency.  World Energy Outlook 2019, Paris, November 2019.  https://iea.blob.core.windows.net/assets/1f6bf453-3317-4799-ae7b-9cc6429c81d8/English-WEO-2019-ES.pdf.

Lewis-Cheatham, S.  Email to J. Ponticello, November 18, 2020.

Ponticello, J.  Email to J.S. Miller, November 18, 2020.

Schuster, J.  SUV Takeover.  LMC Automotive, Detroit, 2018.  https://lmc-auto.com/news-and-insights/segment-trends-cars-and-suvs/.  Accessed November 24, 2020.

U.S. EPA.  How does MOVES Classify Light-Duty Trucks?  Washington, D.C., 2019.  https://www.epa.gov/moves/how-does-moves-classify-light-duty-trucks.  Accessed November 23, 2020.

Voel, T.  “Rise of S.U.V.s: Leaving Cars in Their Dust, With No Signs of Slowing”, The New York Times, May 21, 2020.  https://www.nytimes.com/2020/05/21/business/suv-sales-best-sellers.html#:~:text=%E2%80%9CS.U.V.s%20made%20up%2047.4%20percent,compared%20to%2072%20percent%20now.%E2%80%9D.  Accessed November 24, 2020.

# SCRAP Work

# The following approach illustrates some R coding that can be used in the future if one wanted to simply have siimilar values for different Azones.  Ultimatlely, however, this approach was not used because we realized it is better to have individual data (which are avaialble for the base year) for each Azone.  But the coding may be useful in the future, so we left it here.

library(dplyr)

# Example classification
urban <- c("Arlington","Falls Church","Alexandria","Richmond City")
suburban <- c("Fairfax County","Henrico","Chesterfield","Albemarle")
rural <- c("Wise","Lee","Buchanan","Grayson")

assign_lttrk <- function(geo){
  if(geo %in% urban) return(0.40)
  if(geo %in% suburban) return(0.45)
  if(geo %in% rural) return(0.55)
  return(0.47) # default fallback
}

azones <- c("Fairfax County","Fairfax City","Falls Church","Albemarle","Wise","Lee")

df <- expand.grid(
  Geo = azones,
  Year = c(2019,2045)
) %>%
  rowwise() %>%
  mutate(
    base = assign_lttrk(Geo),
    LtTrkProp = ifelse(Year == 2019, base, base + 0.06)
  ) %>%
  select(Geo, Year, LtTrkProp)

write.csv(df,"azone_lttrk_prop.csv",row.names = FALSE)