File 16_azone_prop_sov_dvmt_diverted.csv

June 4, 2026

Definition of the Input File

VisionEval (https://visioneval.github.io/docs/model-inputs.html#verspm-azone_prop_sov_dvmt_diverted.csv) defines azone_prop_sov_dvmt_diverted.csv as containing one attribute: PropSovDvmtDiverted, defined as the “proportion of household daily vehicle miles traveled (DVMT) in single-occupant vehicle (SOV) tours with round-trip distances of 20 miles or less that are diverted to bicycling or other slow-speed modes of travel.” Changing this value represents a policy or scenario goal, rather than a directly observed quantity, and is specified by Azone and forecast year. For more about the details of how, exactly, this input file is used and how results may be interpreted, see the section under Other Notes.

Possible Scenario

One possible scenario in VisionEval based on this input file is to evaluate how investments in active transportation infrastructure, such as bike lanes, greenways, and pedestrian improvements affect short-distance travel behavior. As described in Task 7.2 Simplified Scenario Development, increasing the proportion of short SOV trips diverted to non-motorized modes can:

- reduce vehicle miles traveled (VMT)

- lower greenhouse gas emissions

- improve public health outcomes

- reduce congestion for short trips

Because this variable applies specifically to short tours (≤20 miles round trip), it targets trips that are most realistically substitutable with biking or walking.  For example, one might develop a scenario where in year 2045, the base condition is that there will be no change in this value—but then a scenario is that there will be a deliberate policy to favor active travel, such as significant investment in infrastructure, improved safety and accessibility for biking and walking, and cultural and behavioral shifts toward non-motorized travel.  In that scenario, the proportion of PropSovDvmtDiverted could be increased from 2024 to 2045 as shown in bold in the figure below.

Method Used for Virginia

Overview

The Azones reflect all 95 counties and 38 independent cities in Virginia. This file specifies a scenario-based policy target for shifting short-distance SOV travel to active modes. Because detailed household travel survey data was not available for all Azones, this implementation uses a stylized, policy-driven approach, assigning low baseline values for the base year (2024) and the forecast year (2045).  Then, in a scenario, users may choose more ambitious targets for year 2045.

Initial Calculations

Unlike other input files, PropSovDvmtDiverted is not directly estimated from observed data, but instead represents a scenario assumption reflecting policy goals and behavioral change potential. The team chose low baseline values of 0.01 for all counties and 0.02 for all cities.  These values reflect limited existing active transportation mode share and higher baseline walking/biking rates in urban areas.  Low values of this (for a baseline case) have been used elsewhere, notably in the RVMPO documentation case on Github (0.04 for 2010 and 0.08 for 2038) and for the Fresno area (roughly 0.01; see RSG, Inc. (2025).

Other Notes

Table 1 shows four runs for a small hypothetical scenario where no inputs were changed except the value of PropSovDvmtDiverted.  In this particular run (based on the household output file with one row for each synthesized household), notice that the output attributes (obtained by summing all rows) for total walk trips, bike trips, and transit trips all increased but only very slightly as PropSovDvmtDiverted increased.   When comparing the top and bottom rows, for example, it is clear that the change in vehicle trips taken by households (4,147) is much larger than the change in walk, bike, and transit trips (a total of 256).  Thus increasing PropSovDvmtDiverted in this case is not assigning those trips to walk, bike, and transit trips but rather appears to be putting them somewhere else.

Table 1.  Examples of VisionEval Runs with Different Values of Diversion

| Input file: azone_prop_sov_dvmt_diverted | Output file: sum of all person trips from all households in Household_SayedMM_2045.csv |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PropSovDvmtDiverted | DVMT | Walk Trips | Bike trips | Transit tripsa | Vehicle trips |
| 0.00 | 217,875 | 78,375 | 429 | 28,571 | 35,043 |
| 0.10 | 215,153 | 78,382 | 430 | 28,589 | 34,631 |
| 0.20 | 212,386 | 78,389 | 430 | 28,607 | 34,211 |
| 1.00 | 190,519 | 78,441 | 436 | 28,754 | 30,896 |

aThese are person trips—that is, with PropSovDvmtDiverted = 0.0, a total of 28,571 transit passenger trips resulted.  If every transit vehicle always had 10 people, then these 28,571 transit passenger trips would be 2,857 transit vehicle trips.

In the same output file is an output attribute for each synthesized household called SovDvmtProp; this attribute is the proportion of household DVMT in single vehicle occupant trips.  For the case of PropSovDvmtDiverted = 0.0, multiplication of SovDvmtProp by the DVMT for each household gives a total of 28,592 DVMT, which is roughly the difference in DVMT between the top and bottom rows of Table 1.

Thus, increasing PropSovDvmtDiverted is leading more trips from other modes, but these are not reflected directly in walk, bike, and transit trips.  Rather, they are reflected in other output attributes.  One of these is the output attribute “PropDVMTDiverted.”  For all households, this value is 0 for the case of PropSovDvmtDiverted=0.0 but is nonzero for all households (which have any values for the output attribute SovDvmtProp) when PropSovDvmtDiverted = 1.  Another attribute is AveTrpLenDiverted which is defined as the “average length in miles of vehicle trips diverted to bicycling, electric bikes, or other 'low-speed' travel modes.”  The sum of this attribute (when PropSovDvmtDiverted = 1) is 50,281—considerably higher than the DVMT differences between the top and bottom rows—the research team expected this difference to be smaller (e.g., 28,592 if all travel were conserved).

As shown above, the resulting file contains one row per Azone for both the base year and forecast year. These values are used by VisionEval to reduce SOV travel demand for short trips and reallocate those trips to active transportation modes. Because this variable represents a policy-sensitive parameter, it is particularly useful for testing the impacts of investments in:

- bicycle infrastructure

- pedestrian improvements

- complete streets policies

In sum, higher values of PropSovDvmtDiverted will result in lower modeled VMT and increased non-motorized travel activity.

Replication Procedure

To recreate this file:

- Define Azones

- Assign baseline diversion rates (2024) based on current conditions

- Assign future diversion targets (2045) based on policy assumptions

- Ensure values are between 0 and 1

- Export to CSV

Example R Script

library(dplyr)
azones <- c("Albemarle County","Charlottesville City")
years <- c(2024,2045)
values <- c(
  0.03, 0.25,   # Albemarle
  0.05, 0.35    # Charlottesville
)
df <- data.frame(
  Geo = rep(azones, each = 2),
  Year = rep(years, times = 2),
  PropSovDvmtDiverted = values
)
write.csv(df, "azone_prop_sov_dvmt_diverted.csv", row.names = FALSE)

How the Input File is Used in VisionEval

This input file is used directly by the VisionEval module DivertSovTravel (located in the VEHouseholdTravel package) and indirectly affects results in the downstream module ApplyDvmtReductions. Within the model workflow, DivertSovTravel is executed after household travel demand has been calculated (i.e., after CalculateHouseholdDvmt, CalculateAltModeTrips, and CalculateVehicleTrips), and before system performance calculations. This means the diversion specified in this file modifies already-estimated household travel behavior rather than influencing initial travel generation.

The key input variable, PropSovDvmtDiverted, is interpreted by the model as a target proportion of short-distance SOV travel (≤20 miles round trip) to be diverted, not as a direct reduction applied uniformly to all households.

The module applies this input in the following steps:

- Identify Eligible Travel

  - For each household, the model estimates the portion of daily vehicle miles traveled (DVMT) that occurs in single-occupant vehicle (SOV) tours with round-trip distances of 20 miles or less (in Urban or Town location types). These trips represent the subset of travel most likely to be shifted to bicycling or other low-speed modes.

- Calculate Azone-Level Diversion Target

  - For each Azone and model year, the total eligible SOV DVMT (from urban and town households only) is summed. This total is multiplied by PropSovDvmtDiverted to determine the total amount of eligible DVMT to be diverted in that Azone.

- Allocate Diversion Across Households

  - The total diversion is distributed back to individual households based on:

    - the amount of eligible SOV DVMT they generate, and

    - the average length of their qualifying trips (shorter trips are more likely to be diverted).

  - This allocation is not uniform; households with more short SOV travel and shorter trips receive a larger share of the diversion.

- Generate Household-Level Outputs

  - The module produces household-level values including:

    - the proportion of DVMT diverted (PropDvmtDiverted), and

    - the average trip length of diverted travel.

  - These outputs are then passed to the ApplyDvmtReductions module, which reduces household DVMT accordingly and reallocates diverted travel to non-motorized modes.  Note, however, that as shown in Table 1, this reallocation may not be evident in the walk, bike, or transit trips—rather, one must look at the DVMTDiverted outputs as well.

An important limitation is that diversion is only applied to households in Urban and Town location types, reflecting the assumption that rural areas have limited feasibility for shifting SOV travel to biking or similar modes.

Interpretation of Input Values

Because this file defines a scenario target rather than an observed rate, the values should be interpreted as the share of eligible short-distance SOV travel that could realistically be shifted under a given policy scenario.

For example:

- A value of 0.05 implies that 5% of eligible short SOV DVMT is diverted.

- A value of 0.25 implies a much more aggressive scenario in which 25% of that eligible travel is shifted to active modes.

Importantly, these values do not mean that all households reduce their travel by that percentage. Instead, the diversion is distributed unevenly across households based on modeled travel characteristics, meaning that some households experience larger reductions than others. As a result, this parameter should be understood as controlling the Azone-level diversion target, rather than specifying a direct behavioral change for individual households.

Sources

- VisionEval Model Inputs Documentation:

https://visioneval.github.io/docs/model-inputs.html#verspm-azone_prop_sov_dvmt_diverted.csv

- VisionEval Scenario Development Guide (Task 7.2): https://visioneval.github.io/docs/scenario-development.html

- VisionEval GitHub Repository: https://github.com/visioneval/visioneval

RSG, Inc.   San Joaquin Valley VisionEval Model Documentation, 2025.  https://www.fresnocog.org/wp-content/uploads/2026/04/SJV_VE_Documentation_Final.pdf