21_bzone_dwelling_units

April 24, 2026

VisionEval (https://visioneval.github.io/docs/model-inputs.html) explains that the definition for bzone_dwelling_units.csv is that it contains three attributes: SFDU, defined as the number of single-family dwelling units; MFDU, defined as the number of multifamily dwelling units which have two or more units; and GQDU, defined as the number of non-institutionalized group quarters dwelling units such as dormitories and military barracks but not jails nor nursing homes.  These values are shown below for four bzones:   515400010001 (an urban area near Charlottesville’s downtown pedestrian mall), 510030107021 (a suburban area near Route 29 in Albemarle County, 510030110004 (a rural residential area in Albemarle County), and 510030109041 (an area near the University of Virginia).  Most bzones have no group quarters dwelling units.

## Possible Scenario

One scenario in VisionEval using this file is to evaluate the impact of targeted housing policy and growth patterns in a specific area.  For example, suppose the City of Charlottesville implements zoning reforms that encourage multifamily development near its downtown core and along transit corridors, while Albemarle County continues to experience primarily single-family suburban expansion. In this case, the model would reflect an increase in MFDU in Charlottesville and a higher share of SFDU in Albemarle. VisionEval would then simulate how these differing development patterns influence household behavior specifically, lower vehicle ownership rates and increased walking, biking, and transit use in Charlottesville, compared to higher car dependence and longer trip distances in Albemarle County.

This type of scenario is particularly relevant for real-world planning decisions, such as evaluating the transportation impacts of comprehensive plans or housing initiatives. For instance, increasing multifamily housing near employment centers or the University of Virginia could reduce short car trips and shift travel toward active modes, while also affecting parking demand and congestion. Similarly, areas with significant group quarters populations such as student housing or assisted living facilities—may exhibit lower vehicle ownership and distinct travel patterns compared to traditional households. By adjusting SFDU, MFDU, and GQDU across Bzones, VisionEval allows planners to directly test how land use decisions translate into measurable changes in VMT, emissions, and transportation system performance.

Note that when adjusting dwelling units, one must also adjust three population files (azone_hh_pop_by_age.csv, azone_gq_pop_by_age.csv, and azone_hhsize_targets.csv).

## Method Used for Virginia

### Base Year

Bzones represent small geographic units (generally Census block groups). This file estimates the number of dwelling units in each Bzone for both the base year (2024) and forecast year (2045).

The methodology follows a Census-based classification approach, using housing unit data and standard definitions of structure type. Where necessary, simplifying assumptions were applied to ensure full coverage and consistency across all Bzones.

Three Census tables were used.

- One source is the American Community Survey (ACS) 5-year estimates, specifically the “Units in Structure” table (Table B25024) for year 2024. This table provides counts of housing units by structure type, which can be directly mapped to VisionEval categories. Single-family dwelling units (SFDU) are defined as structures with one unit, while multifamily dwelling units (MFDU) include all structures with two or more units (e.g., duplexes, apartments, large multifamily buildings).  (For two categories of housing, assignment to multifamily or single family was not clear.  The research team assigned mobile homes to multifamily dwelling units given their closeness to each other but assigned “boat, RV, van, etc.” to single family dwelling units, thinking that these would tend to generate a larger number of trips comparable to single family dwelling units.

- The second table was Table P5 from the 2020 decennial census.  This table gives the population in group quarters dwelling units that are not institutionalized—that is, military barracks, dormitories, and others—but excluding skilled nursing facilities and prisons.  The research team could not identify a group quarters table from the 2024 ACS.

- The third table was Table B25024 for year 2020.

Then, the 2024 group quarters units were determined by multiplying the 2020 group quarters units by the ratio of 2024 single and multifamily dwelling units to the 2020 single and multifamily dwelling units.  For example, for block group 510030106031, there were 363	single family and 754 multifamily dwelling units in 2024, for a total of 1017 non-GQ dwelling units.  The 2020 data showed a smaller number of non-GQ dwelling units (942) and 60 GQ dwelling units.  Thus, the ratio of this change from 2024 to 2020 was applied to the 60 group quarters units in 2020 to estimate 71 group quarters dwelling units in 2024.

1017 SFDU and MFDU in 2024942 SFDU and MFDU in 202060 GQDU in 2020=71 GQDU in 2024

Forecast year

Data were acquired from Woods & Poole (2024) which give changes in total households for 2045 by Azone (city and county).  The change from 2024 to 2025 for this value was applied to the 2024 data.  (Note that in some cases, Woods & Poole aggregates a city and county; for example, Charlottesville City (FIPS code 51540) and Albemarle County (FIPS code 51003) are the same Woods & Poole are (51901).

For example, bzone 510030106031 is located in Albemarle County.  Woods & Poole (2024) reports that the number of households in the combined area of Charlottesville and Albemarle County is forecast to grow from 66,064 in 2024 to 78,497 in 2045.  This rate of increase (18.82%) was applied to the 2024 data to give the 2045 values shown below.

| Year | SFDU | MFDU | GQDU |
| --- | --- | --- | --- |
| 2024 | 363 | 754 | 71 |
| 2045 | 431 | 896 | 84 |

Reference

Woods & Poole, Inc. Technical Description of the Woods & Poole Economics, Inc. 2024 Regional Projections and Database, Washington, D.C., 2024.

## Appendix—Other Notes

The resulting file contains one row per Bzone for each modeled year. These values are used by VisionEval to:

- estimate household distribution

- allocate population across housing types

- model vehicle ownership and travel behavior

Because housing type is strongly correlated with travel demand, accurately representing the balance between single-family and multifamily units is critical for realistic model outputs.

Note for running VisionEval: if total SFDU + MFDU + GQDU is 0 for a given bzone, only SFDU is set to 1 to prevent crashing. Bzones are expected to have at least one usable dwelling unit.

## Replication Procedure

To recreate this file:

- Obtain Census housing data (units in structure) at the block group level

- Classify units into SFDU (1-unit structures) and MFDU (2+ unit structures)

- Obtain group quarters population data

- Assign GQDU using a 1:1 population-to-unit assumption

- Map all values to Bzones

- Assign values for each modeled year

- Export to CSV

### Example R Script

library(dplyr)

df <- data.frame(
  Geo = c("515400002011","510010902022","510010902023"),
  Year = c(2045,2045,2024),
  SFDU = c(307,1097,719),
  MFDU = c(449,148,134),
  GQDU = c(0,0,35)
)

write.csv(df, "bzone_dwelling_units.csv", row.names = FALSE)

Locations of Materials Internal to VTRC

- C:\Users\John.Miller1\Commonwealth of Virginia\TM-VDOT-VTRC EPE - Documents\TeamStuff\EPEStuff\DataSets\WoodsPoole\Woods&Poole2024\SP_2024_DC_MD_VA.zip\WP_Download_2024\SP_2024_DC_MD_VA

- https://covgov.sharepoint.com/sites/TM-VDOT-vtrcepe/Shared%20Documents/TeamStuff/ProjectStuff/Summer2024/VTrans/2024%20Local%2065+Population.xlsm