Explanation of how to get data for 23_bzone_hh_inc_qrtl_prop.csv

April 6, 2026

Definition of the Input File

VisionEval (https://visioneval.github.io/docs/model-inputs.html) explains that this file has “the proportion of households in 1st, 2nd, 3rd, and 4th quartile of household income.”  More specifically, these are proportions are intended to be by Azone (see https://visioneval.github.io/docs/model-inputs.html#verspm-household-proportion-by-income-bzone_hh_inc_qrtl_prop.csv).  For example, consider Bzone 515400010001 which is in the City of Charlottesville (Azone 540).  The data below suggest that 28.8% of the households in this bzone are in the lowest quartile of income for the City of Charlottesville and that 15.8% of the households in this bzone are in the highest quartile of incomes for the City of Charlottesville).

Data for two other Bzones in a different Azone (003, Albemarle County) are also shown.  For Bzone 510030107021 (a suburban area near Route 29 in Albemarle County with some employment), just 3% of these households are in the highest quartile of incomes for Albemarle County.  By contrast, for Bzone 510030110004 (a rural residential area in Albemarle County), 25.9% of the households are in the highest quartile of incomes for Albemarle County.

Possible Scenario

The document Task 7.2 Simplified Scenario Development guide, available at A Playbook for Simplified Approaches for Creating Scenarios in VisionEval, does not give an explicit reason for modifying this input file.  However, the VisionEval documentation (https://visioneval.github.io/docs/model-inputs.html#verspm-verspm-predicthousing) explains that these income quartiles “help VisionEval assign households to a compatible Bzone location.”  Accordingly, one possible scenario is to examine how demand by Bzone changes due to uncertainty—e.g., one could assess, for instance, how changes in these quartiles for a given Bzone influence total model outputs, such as demand for transportation modes.

Method Used for Virginia

Users may wish to replace the statewide values used for Virginia with specific values.  In theory, the quartiles should be tied to each Azone.  In practice, the research team tied the quartiles to a statewide household income.  The following approach was used:

- Obtain Census data table B19001 for the five year dataset of 2020, 2021, 2022, 2023, and 2024).

- Find the number of households in the first, second, third, and fourth income household quartiles.  For example, in Virginia, these are roughly as follows:

  - FROM $0 to TO half the households in the range of $45,000 to $49,999 (first quartile)

  - FROM half the households in the range of $45,000 to $49,999 TO three-quarters of the households in the range of $75,000 to $99,999.

  - FROM one-quarter of the households in the range of $75,000 TO $99,999 to two-thirds of the households in the range of $150,000 to $199,999.

  - FROM one-third of the households in the range of $150,000 to $199,999 TO infinite.

- Convert these numbers to proportions for each Bzone and round them to three decimal places.

- If the proportions sum to more or less than 1.000, then arbitrarily adjust the largest proportion by 0.001 such that the proportions sum to exactly 1.000.

- For Bzones with no households, make the proportions each 0.250.

The calculations are in the file 23_bzone_hh_inc_qrtl_prop.xlsx with the data being in the file 23_bzone_hh_inc_qrtl_prop.csv

Scrap Work

Chapter 7 Model Inputs | VisionEval User Guide

Chapter 7 Model Inputs | VisionEval User Guide