File 24_bzone_lat_lon

March 30, 2026

## Definition

VisionEval (https://visioneval.github.io/docs/model-inputs.html) explains that this file contains the centroid x (longitude) and centroid y (latitude) points for each census block group from the years 2019 through 2024. The data is from the US Census Bureau TIGER/Line Geodatabases.   These values are shown below for three bzones:   515400010001 (an urban area near Charlottesville’s downtown pedestrian mall) , 510030107021 (a suburban area near Route 29 in Albemarle County with some employment), and 510030110004 (a rural residential area in Albemarle County).

Table

| 515400010001 | 2024 | 38.01793 | -78.4697 |
| --- | --- | --- | --- |
| 510030107021 | 2045 | 38.10817215 | -78.53293991 |
| 510030110004 | 2045 | 38.13576186 | -78.49172758 |

Note: The year-to-year changes are not present in this file, as it is a geographical data point.

Method

The state-level geodatabase for Virginia was downloaded, then the following process was followed for each year:

- In ArcGIS, open the Block_Group Feature Class in a new map.

- Use the tool Calculate Geometry Attributes to add the centroid x- and y-coordinates for each block group to the attribute table using Decimal Degrees

- Edit the fields of the attribute table to create a new field for Year, then only make the following fields visible: GEOID (Alias: “Geo”), Year, Latitude, and Longitude

- Add the relevant year to the ‘Year’ field for all rows using the tool Calculate Field

After completing this for every desired year, export the attribute tables to Excel and combine the data into one table.

Here is a snapshot of the file: