37_ marea_speed_smooth_ecodrive.csv

May 26, 2026

Definition of the Input File

VisionEval (https://visioneval.github.io/docs/model-inputs.html) explains that this file tells one the extent to which there is speed smoothing.

- For the first two attributes (FwySmooth and ArtSmooth) this number ranges from 0 (no deployment) to 1 (full deployment) of a system, such as a variable speed limit system or some other type of system, to manage traffic flow.  (This presumably does not cover ramp metering which is part of a different input file (marea_operations_deployment.csv).  As pointed out in the later Task 7.2 Simplified Scenario Development guide, connected vehicle deployments may affect this value as well, although that document notes connected vehicles may be included a third input file (other_ops_effectiveness.csv)

- The next two attributes are based on “policies with individuals committed to driving less aggressively with fewer harder braking or fast acceleration activities” where the 0 means percentage of drivers who agree to “drive in this manner” for freeways and arterials.

For example, the table below suggests that 10% of freeways in the City of Fredericksburg have a deployment of a speed smoothing system.

Possible Scenario

The aforementioned document Task 7.2 Simplified Scenario Development guide available at A Playbook for Simplified Approaches for Creating Scenarios in VisionEval mentions this input file as one of three one can use to “evaluate changes in policy and physical interventions to improve congestion through improved speed management and reduced delays.”   (The other two input files are marea_operations_deployment.csv and other_ops_effectiveness.csv).  Thus, presumably a scenario could be to evaluate how deployment of adaptive speed limit systems reduces fuel consumption compared to, say, a reduction in the number of trips or possibly an increase in lane-miles (if they latter reduces idling).

Method Used for Virginia

For the first two attributes, the research team identified one news article that discusses implementation of variable speed limits in Caroline, Spotsylvania, and Fredericksburg.  That article is https://improve95.vdot.virginia.gov/variable/ and is titled “I-95 North variable speed limits.”  The figure of 0.10 was chosen to indicate that a nonzero portion of freeways have a variable speed limit.  Otherwise, however, the research team is not aware of firm evidence showing freeway and arterial smoothing in Virginia.

For the second two attributes, the research team could not find evidence of Virginia drivers practicing ecodriving.  This practice certainly is possible; for example, a VTTI study, cited at https://www.claimsjournal.com/news/national/2020/09/24/299537.htm where a news clip was titled “Study Shows Monitoring Plus Coaching Can Cut Risky Driving in Half” shows that drivers of heavy duty vehicles, such as trucks, have a safety incentive to perform some ecodriving.  Thus a nonzero value is possible.  However, for the purposes of setting a baseline, the values in this input file are set to zero; the user may adjust these as more data become available.