Optional File 43_other_ops_effectiveness.csv

Definition of the Input File

VisionEval (https://visioneval.github.io/docs/model-inputs.html) explains that this file is used only if one wants to “to modify the other delay effects of operations in different road classes”, however, note further that this file is not needed unless one is considering a new technology besides those mentioned in marea_operations_deployment and marea_speed_smooth_ecodrive.  For example, if one is using ramp metering, one would not use this file because ramp metering is covered in marea_operations_deployment.

The values below show that no technologies beyond those noted in the other two input files (marea_operations_deployment and marea_speed_smooth_ecodrive) are being used to address recurrent or non-recurrent delay on arterials and freeways.

Possible Scenario

The document Task 7.2 Simplified Scenario Development guide available at A Playbook for Simplified Approaches for Creating Scenarios in VisionEval mentions this input file as one of three one can use to “evaluate changes in policy and physical interventions to improve congestion through improved speed management and reduced delays.”   (The other two input files are marea_operations_deployment.csv and marea_speed_smooth_ecodrive.csv).

Thus this file is potentially useful for developing a scenario where one uses technology not specified in the other two files to improve operations.  For example, the Task 7.2 document describes a scenario where a “global connected vehicle program” is forecast to reduce “recurrent congested delay” by 30% and “nonrecurrent delay” by 70%.  In that case, one changes the zero values shown in the yellow boxed cells to 30 and 70.  One challenge with this file is defining how a technology would impact various delay levels—in this example, it is imagined that the connected vehicle program has delay reductions for the two worst delay levels (severe and extreme) but not the three better cases (no delay, moderate delay, and heavy delay).

Method Used for Virginia

For the base case, it is assumed that there are no operational tools (beyond those mentioned in the two other input files marea_operations_deployment and marea_speed_smooth_ecodrive) and thus values of 0 are presumed.